SOME SAMPLE QUESTIONS

Note: The purpose of this document is to give you an idea of the questions on the quiz.

Assembly Programming Introduction

1. Assume the following values are stored at the indicated memory addresses and registers:

Address 	Value    	Register 	Value

0x100     	0xFF      	%rax        	0x100

0x104    	0xAB      	%rcx       	0x1

0x108     	0x13       	%rdx      	0x3

0x10C     	0x11

Fill in the following table showing the values for the indicated operands:

Operand			Value			Answer	Comment

%rax				__________		0x100    	Register

0x104				__________		0x AB       	Absolute address

$0x108				__________		0x108     	Immediate

(%rax) 				__________		0xFF         	Address 0x100

4(%rax) 			__________		0xAB       	Address 0x104

9(%rax,%rdx) 			__________		0x11     	Address 0x10C

260(%rcx,%rdx) 		__________		0x13     	Address 0x108

0xFC( ,%rcx,4) 			__________		0xFF		Address 0x100

(%rax,%rdx,4) 			__________		0x11		Address 0x10C

Command Suffix (Optional feature):

Example MOV instruction:

2. For each of the following lines of assembly language, determine the appropriate instruction suffix based on the operands. (For example, mov can be rewritten as movb, movw, movl, or movq.)

mov_ %eax, (%rsp)			l

mov_ (%rax), %dx			w

mov_ $0xFF, %bl			b

mov_ (%rsp,%rdx,4), %dl		b

mov_ (%rdx), %rax			q

mov_ %dx, (%rax)			w

3. Each of the following lines of code generates an error message when we invoke the assembler. Explain what is wrong with each line. (Assume 64 bit machine)

movb $0xF, (%ebx)			Cannot use %ebx as address register (addresses are 8 bytes long)

movl %rax, (%rsp)			Mismatch between instruction suffix and register ID

movw (%rax),4(%rsp)  		Cannot have both source and destination be memory references

movb %al, %sl 				No register named %sl

movq %rax, $0x123			Cannot have immediate as destination

movl %eax, %rdx			Destination operand incorrect size

movb %si, 8(%rbp)			Mismatch between instruction suffix and register ID

4. LEA Command Examples:

Suppose register %rax holds value x and %rcx holds value y. Fill in the table below with formulas indicating the value that will be stored in register %rdx for each of the given assembly-code instructions:

leaq 6 (%rax) , %rdx					6+x

leaq (%rax, %rcx) , %rdx				x + y

leaq (%rax, %rcx,4), %rdx				x +4y

leaq 7 (%rax, %rax, 8) , %rdx				7 + 9x

leaq 0xA(,%rcx,4), %rdx				10 + 4y

leaq 9(%rax,%rcx,2), %rdx				9 + x + 2y

5. Find the C expression t:

long scale(long x, long y, long z) {

long t = ______________________

return;

When

long scale(long x, long y, long z)

x in %rdi, y in %rsi, z in %rdx

scale:

leaq (%rdi,%rsi,4), %rax		x + 4*y

leaq (%rdx,%rdx,2), %rdx		z + 2*z = 3*z

leaq (%rax,%rdx,4), %rdx		(x+4*y) + 4*(3*z) = x + 4*Y + 12*Z

ret

********************